- K0_empirical.py: This script imports the Chestatee DEM, takes values of E (given as the 10Be mean catchment erosion rate), m_sp (0.503) and n_sp (1.224); and calculates an empirical value of K0 using the equation:

K0 = Ei=1i=jAim Sin	(5)

Where j is the number of grid nodes. I calculate values of K0 using the recalculated mean catchment erosion rate of Reusser et al (7.96 E -6) as well as the mean value plus / minus the standard deviation (1.77). The script creates an output folder and saves the following values to a csv file:

K0 Values

- K0_mean = 8.2912443616803223e-07

- K0_min = 6.4475882661810545e-07

- K0_max = 1.013490045717959e-06

Some thoughts:

- The implication of the equation above is that given a measured mean catchment erosion rate and reasonably constrained values of m and n, you can calculate an effective mean fluvial erodibility of a watershed in steady-state. In the future, this could be expanded to constrain values of U/Ksp  and explore dynamic topographic responses.