- Create_Terrains_to_Verify_D_star.py: Starts with the real DEM, takes values of Ksp and D*, and runs to near steady state. Dt = 1E6 years, tmax = 1E9 years. I run the script multiple times, each time using the same value of Ksp but changing the value of D*. Creates a folder called “Create_Terrains_to_Verify_D_star_output”. Structure inside this folder:

  - DEM_Image: Elevation map of the model grid exported at specified timesteps. These outputs are not functional, but rather just informational to ensure quality control.

  - DZDT_Map: Rate of elevational change map exported at specified timesteps. These outputs are not functional, but rather just informational to ensure quality control. By t=1E9 years, the maps show that all model grids are very close to steady-state.

  - Export_DEM: Model grids exported at specified timesteps. These are not functional, but rather just informational to ensure quality control.

  - Final_DEMs: Final model grids exported at the end of each run. The exported grids are automatically named for their unique D* value. The D_star_Calib.m file uses this folder and its contents for input data.

Since they are not functional, the DEM_Image, DZDT_Map, and Export_DEM folders can be deleted or overwritten between runs. Only the Final_DEMs folder should be preserved between runs. I discovered that Ksp = 5E-7 works best (see below). Preliminary experiments showed that best-fit D* values were somewhere between 100-400. Therefore, I created terrains with D* values between 100-400 at increments of 25 (n = 13).

- D_star_Calib.m: Loops through all grids in the Create_Terrains_to_Verify_D_star_output/Final_DEMs folder and compares their slope-area relationships relative to the real DEM. It also creates an output folder called “D_star_Calib_output” in which it stores informational graphics. The value of Ksp used in the Create_Terrains_to_Verify_D_star.py script translates the slope area relationship along the Y-axis. Preliminary experiments showed that a K value of 5E-7 best fits the Y-positions of the real DEM. Therefore, all terrains in the Final_DEMs folder were created using Ksp = 5E-7.

I compared the slope-area relationship of the real Chestatee DEM with the slope-area relationships of terrains with D* values ranging from 100-400 (increments of 25, n = 13). Terrains were considered valid matches when the slope-area relationship rollover occurred between a drainage area of 1 and 2 pixels (i.e. maximum slope occurs at 2 pixels, second highest slope occurs at 1 pixel, third highest slope occurs at 3 pixels). Valid matches occurred at D* values between 200-325 (n = 6). Therefore, the following values will be used:

D* Values

- D*_mean = 262.5

- D*_min = 200

- D*_max = 325

Some thoughts:

- Since the rollover occurs at a drainage area between 1-2 pixels, this means that hillslope processes are only important at a spatial scale close to or only slightly larger than the DEM resolution. In other words, at the resolution of this DEM, fluvial processes are by far the dominant process, and hillslope processes probably play a negligible role in shaping the landscape at this scale. This is handy, because after setting a value of D*, we can probably ignore the effects of variable D* values on the model outcomes.