Klc_validate.py: The purpose of this script is to estimate a “modern” (i.e. 20th / early 21st century) mean sediment flux (Qs, m3 yr-1) and mean catchment erosion rate (E1rc, m yr-1) using a rating curve. E1rc will serve as an independent estimate of the mean catchment erosion rate that can be compared / contrasted with the value (E1) suggested by the K1lc array. The script:

  - 1. Constructs a sediment rating curve ( log(SSC) = m log(Qw) + b ) using data from 02333500_Sed.csv. Separate rating curves are constructed for each discreate decade of data, but the rating curve that is ultimately used for this analysis is created using data from all decades.

  - 2. Uses the rating curve to estimate a daily sediment flux for each day in the 02333500_Hydro.csv record [ Dates : {19300101 - 19320131}, {19400401 - 20211231} ].

  - 3. Calculates a total sediment flux for each year [ {1930 - 1932}, {1940 - 2021} ] in the 02333500_Large.csv record. This produces the Qs_annual array.

  - 4. Calculates an array of annual mean catchment erosion rates (mcer array) by dividing the Qs_annual array by catchment area.

  - 5. Calculates the mean of the Qs_annual array (Qs_annual_mean) and the mcer array (E1rc_mean).

  - 6. Calculates a minimum and maximum constraint on Qs and E1rc (Qs_annual_min, Qs_annual_max, E1rc_min, E1rc_max) using the interquartile range of the Qs_annual and mcer arrays.

- 02333500_Sed.csv: USGS daily water discharge data from gauge 2333500. Column “36255_00060_00003” is daily average discharge in cfs.

- 02333500_Hydro.csv: USGS water quality data from gauge 2333500.

Qs, Rating curve implied sediment export values (m3 yr-1)

- Qs_annual_mean = 13199.80563

- Qs_annual_min = 3899.409751

- Qs_annual_max = 14802.56747

E1rc, Rating curve implied mean catchment erosion rate values (m yr-1)

- E1rc_mean = 3.33102841064183E-05

- E1rc_min = 9.84033024739712E-06

- E1rc_max = 3.73549233781876E-05